面向机器人辅助肩关节置换的骨表面实时可视化研究

课题背景

随着医学影像、手术导航和机器人技术的发展，骨科手术正在由主要依赖术者经验的操作方式，逐步转向基于患者影像、术前规划和术中导航的精准手术方式。此类系统通常根据患者 CT 影像重建骨骼三维模型，并在模型上规划植入物位置、骨切削范围和手术工具的安全边界。手术过程中，系统还需要将术前规划与患者实际骨骼进行配准，持续跟踪手术工具，并向术者显示计划的执行情况。

近年来出现的 Mako Shoulder 等机器人辅助肩关节置换系统，为这一研究方向提供了较典型的应用场景。在反式肩关节置换中，机械臂可在触觉边界约束下辅助完成肩胛盂基板骨床的准备，包括肩胛盂表面磨削和中央固定区域的加工。公开手术视频显示，磨钻过程中系统会在三维肩胛骨表面显示计划去除区域，并根据实际操作逐步更新显示结果，使术者能够判断当前磨削位置、剩余处理区域和计划完成程度。

因此，这一场景中的三维可视化并不是静态显示术前 CT 模型，而是需要根据磨钻工具的连续运动，实时反映骨表面的几何变化，并将当前表面、目标表面、工具位置、剩余磨削量和潜在风险统一表达出来。这一问题涉及医学影像三维重建、曲面表示、动态几何处理、实时计算、手术导航和医学可视化等多个研究领域。

CT 三维重建、手术规划、植入物模型、导航显示和术后评估通常都以 surface/mesh 为主要数据形式。表面模型能够直接描述骨骼边界及其局部几何特征，也便于与植入物 CAD 模型、手术计划和常用三维渲染流程衔接。因此，本课题将以 surface 作为基本几何表示，重点研究骨磨削过程中的表面更新、误差计算和实时可视化。

课题拟研究的主要内容

本课题拟以机器人辅助肩关节置换中的肩胛盂磨削为主要应用背景，以 CT 重建得到的三角表面模型为输入，研究手术磨钻作用下骨表面的动态表达和可视化问题。主要内容包括：

- 手术场景与可视化需求分析。 调研肩关节置换中的术前规划、患者—影像配准、工具跟踪、机械臂约束和术中显示流程，明确磨钻过程中需要表达的几何信息和状态信息。

- 动态骨表面表示与更新。 研究适合骨磨削过程的 surface 表示方法，以及如何根据磨钻工具的位置、姿态和运动轨迹，对局部骨表面进行连续更新，并保持表面连续性、几何精度和网格质量。

- 实时计算与显示。 研究局部表面相交检测、受影响区域筛选、曲面更新、法向计算和显示的高效实现，提高从工具运动到可视化反馈的实时性；可将 CUDA、GPU 并行计算或计算着色器作为其中的加速手段。

- 计划执行状态与风险表达。 研究如何比较当前骨面和计划骨面，并在三维表面上显示尚需去除的区域、剩余磨削深度、计划完成程度、可能的过度磨削以及工具与安全边界的关系。

- 方法评价。 从几何误差、计算效率、网格质量、连续运行稳定性和可视化效果等方面，对所研究方法进行实验验证，并与离线高精度结果或已有 surface 处理方法进行比较。

可重点探索的研究方向

课题不预先限定唯一的技术路线。学生可在完成文献调研和初步实验后，从以下方向中选择一个作为主要研究重点：

- 骨表面的增量更新与实时实现： 研究工具连续运动条件下局部骨表面的快速更新、数据组织与实时显示，并可结合 GPU 加速关键计算环节。

- 动态表面的局部重建与质量保持： 研究重复磨削后产生的退化三角形、裂缝、法向不连续和拓扑变化问题。

- 计划误差和不确定性可视化： 研究当前表面与计划表面的差异，以及配准、跟踪、工具标定和系统延迟误差的视觉表达。

- 多分辨率和自适应 surface 表达： 根据骨面曲率、计划边界和工具作用范围调整局部表面分辨率，在几何精度、网格规模和实时性之间取得平衡。

最终的具体研究问题和创新点，应由学生在前期调研基础上进一步凝练。

课题范围

硕士阶段建议首先以肩胛盂局部区域、刚性骨表面和球形磨钻为主要研究对象，重点解决几何更新和术中可视化问题，不同时承担机器人控制、软组织形变、完整手术仿真和临床系统认证。任意复杂网格和任意工具的通用布尔处理可以作为后续扩展，而不作为完成课题的必要条件。

在肩胛盂场景得到验证后，相关方法还可进一步向髋臼、膝关节、脊柱和其他骨磨削手术推广。

预期成果

- 机器人骨磨削实时表面可视化的文献和需求调研；

- 一种面向该场景的动态 surface 表示或更新方法；

- 一个能够读取骨表面、手术计划和工具轨迹的可视化原型；

- 实时计算与显示方面的实现及性能分析，其中可包含 GPU 加速模块；

- 与离线高精度结果或已有 surface 处理方法的对比实验；

- 面向肩胛盂磨削任务的精度、效率和稳定性评价；

- 相关学术论文和硕士学位论文。

参考文献

- Sanchez-Sotelo J, Marigi EM. Robotic-assisted baseplate preparation in primary reverse total shoulder arthroplasty: surgical technique and contemporary review. JSES Reviews, Reports, and Techniques. 2025. https://doi.org/10.1016/j.xrrt.2025.08.013

- Athwal GS, Nelson A, Antuna S, et al. Glenoid preparation in reverse shoulder arthroplasty: robotic arm-assisted preparation compared to manual preparation and patient-specific guides. Journal of Shoulder and Elbow Surgery. 2025;34:2022–2030. https://doi.org/10.1016/j.jse.2024.12.007

- Crowther LJ, Nadzadi ME. Accuracy of robotic-arm assisted reverse shoulder arthroplasty. ORS 2026 Annual Meeting Paper No. 2155. https://www.ors.org/wp-content/uploads/AM26/2155.pdf

- Menendez ME, et al. Early adopter surgeon experience with Mako robotic-assisted reverse shoulder arthroplasty: perceived utility, workload, and workflow reliability. Journal of Shoulder and Elbow Surgery. 2026. https://doi.org/10.1016/j.jse.2026.05.016

- Mahmoud AH, Porumbescu SD, Owens JD. Dynamic Mesh Processing on the GPU. ACM Transactions on Graphics. 2025. https://doi.org/10.1145/3731162

- Schmidt R, Brochu T. Adaptive Mesh Booleans. 2016. https://arxiv.org/abs/1605.01760

- Nehring-Wirxel J, Trettner P, Kobbelt L. Fast Exact Booleans for Iterated CSG using Octree-Embedded BSPs. Computer-Aided Design. 2021. https://arxiv.org/abs/2103.02486

- Gong X, Feng H-Y. Cutter-workpiece engagement determination for general milling using triangle mesh modeling. Journal of Computational Design and Engineering. 2016;3(2):151–160. https://doi.org/10.1016/j.jcde.2015.12.001

- Karras T. Maximizing Parallelism in the Construction of BVHs, Octrees, and k-d Trees. High Performance Graphics. 2012. https://research.nvidia.com/publication/2012-06_maximizing-parallelism-construction-bvhs-octrees-and-k-d-trees

- Stryker. Mako Shoulder product and workflow information. https://www.stryker.com/us/en/joint-replacement/systems/mako-shoulder.html

相关视频

- VuMedi：Mako Shoulder Robotic-Arm Assisted Surgical Video。https://www.vumedi.com/channel/stryker/tab/mako-shoulder/video/mako-shoulder-robotic-arm-assisted-surgical-video/

- Mako 肩胛盂准备流程与视频入口：https://shoulderelbow.org/2025/03/28/stryker-mako-robot-assisted-reverse-shoulder-replacement/